Commercial Fishing Vessel Safety Examination Checklist

State Registered

26 to less than 40 feet in length

Inside Boundary Line

Waters Inside Coastal

| GENERAL VESSEL REQUIREMENTS |  |  |  |
| --- | --- | --- | --- |
| Vessel Name: |  | Number: |  |
| # BRIDGE & DOCUMENTS |  |  |  |
| 33 CFR 173 | Registration/Markings: Original on board, current; Vessel’s number on both sides of forward part of hull. Contrasting color |  | O Yes O No O N/A |
| 46 CFR 28.165 | Injury Placard (All Vessels) |  | O Yes O No O N/A |
| 33 CFR 155.450 | Oil Pollution Placard (Vessels > 26 Feet) |  | O Yes O No O N/A |
| 33 CFR 151.59 | MARPOL (Garbage) Placard (Vessels > 26 Feet) |  | O Yes O No O N/A |
| 33 USC 1602 33 USC 2020 72 COLREGS | Navigation Lights: Required during darkness and/or in or near areas of restricted visibility (fog) Side Lights (112.5o) All around white light or masthead (225 o and sternlight 135 o) Fishing lights Red over White (360o) 1 meter apart, required on all fishing vessels except TROLLERS. Trawlers use green over white. |  | O Yes O No O N/A |
| ##### 33 USC 1602 33 USC 2020 72 COLREGS | ##### Dayshapes Required fishing dayshape, two black cones apex to apex. Not required on TROLLERS. |  | O Yes O No O N/A |
| ##### 33 USC 1602 33 USC 2020 72 COLREGS | Sound Producing Devices: Vessels < 12m (39.4 ft): Means of Making an Efficient Sound Signal 12m < 20m (65.6 ft) a powered whistle |  | O Yes O No O N/A |

| ###### LIFESAVING |  |  |
| --- | --- | --- |
| 46 CFR 28.110 ##### 46 CFR 28.135 ##### 46 CFR 28.140 | ##### Personal Flotation Devices (PFDs) Type I, II, III, V or Immersion Suit. One for each person on board. Marked with name and retroreflective tape Properly maintained in good working order and in good shape Stowed readily accessible | O Yes O No O N/A |
| 46 CFR 28.115 46 CFR 28.135 | Ring Life Buoys: Required 1 orange ring life buoy, 24 inches in diameter. a. Marking with name and retroreflective tape b. Ring buoy is required to have 60 feet of line attached. c. Stowed readily accessible in good working condition. | O Yes O No O N/A |
| # ENGINE ROOM/MISCELLANEOUS |  |  |
| ##### 46 CFR 28.155 46 CFR 28.160 46 CFR 25.30 | Portable Fire Extinguishing Equipment: Required 2 B-I Fire Ext on board. Approved by USCG, UL or FM. Good Condition/Pressure gauge OK or weighed and tagged if no gauge. Mounted properly in bracket | O Yes O No O N/A |
| ##### 46 CFR 25.35 | Flame Arrestor (gas power) Approved and in good condition | O Yes O No O N/A |
| ##### 46 CFR 25.40 | Ventilation (gas power) Adequate/Working ventilation to engine compartment | O Yes O No O N/A |
| ##### 33 CFR 159.7 | Marine Sanitation Device: Required only if there is an INSTALLED toilet. Type III: Holding tank to prevent overboard discharge. Overboard valve must be secured | O Yes O No O N/A |
| ##### POLICY | Excess Safety Equipment: Any safety equipment (PFDs, EPIRBs, Survival Craft, Fire Extinguishers, Visual Distress Signals) that is in excess than what is required on board must meet one of the following: - Serviced and properly maintained in good working order, - Marked “For Training Only” and kept separate from serviceable items, or - Removed from the vessel. |  |

Vessel Name: _____________________				Official Number: __________________

Owner:________________________________________

Owner Address:_________________________________

_________________________________

Owner Phone:___________________________________

Color: Hull ___________	Trim ____________	Superstructure ___________

Hull Type:  Wood  Alum  FRP  Steel

Fuel Type:  Gas  Diesel / Portable   Fixed

Propulsion:  Outboard  Inboard  In/Out-board

Horsepower: __________________

Fishing Equipment:  Long Line  Multi-Rig  Troll        Gill Net  Trap

Other: __________________

Route: Inland  WIC 	/       <3NM  <12NM

Max POB: _____

Examiner Name: _______________________________________

Date of Exam: _____/_______/_2021________

CFVS Decal #: 21 - ______________________

Issued vessel D13 Immersion Suit Exemption Letter?  Yes  No